|  | Laboratory Assignment Number 8 for ME 218b Due by 5:00 pm on February 5, 2026 |
| --- | --- |

Part 1: A Simple Mobile Platform

Assignment:

You are to design and build a mobile platform that will be able to begin in a random orientation at position A. There will be a beacon placed approximately as shown in the diagram below. The beacon will pulse infrared light at a frequency of 1427Hz.  Your droid should query the Command Generator over SPI (protocol and command descriptions in Appendix A) and follow the commands to position itself. Get a sign-off from a TA/Ed/lab coach after demonstrating your completed vehicle.

|  | 1.1) | Your vehicle must be constructed from scratch using the motors and wheels provided and carry the PIC32, Command Generator and other electronics of your choosing. It should use a tether to supply power and may connect to the PC to display debugging messages. The motors and wheels may not be modified and must be returned in good working order at the end of the lab. Note1: the motor leads are somewhat fragile. Please be sure to strain-relieve them to prevent fatiguing the motor connections. |
| --- | --- | --- |

In the report:

The report should include a description of the mechanical hardware design, electrical hardware design (schematics and design calculations), along with state charts and pseudo-code for the software. Additionally, include a narrative description of the hardware and software (brief and to the point, max. 1 page each), and a Highlighted listing of the final software that was implemented. When copying your code into your report, please use the Highlight program to prepare an RTF file using the “bright” color theme. You can then open the RTF file in Word and copy and paste the code into your report and it will maintain the color highlighting. Look at the code that you paste into your report. If the indenting is messed up anywhere, it means that you had tab characters in your source files. If the indenting is messed up, clean this up before running Highlight again and submitting your report.

Schematics

Table of Pins

Motor Output (OC, PWM)

| Controlled Motor | Pin# on PIC32 | Port on PIC32 | PWM Channel PIC32 | Pin# on L293NE | Port on L293NE |
| --- | --- | --- | --- | --- | --- |
| M1+ | 11 | RB4 | 1 | 2 | 1A |
| M1- | 14 | RB5 | 2 | 7 | 2A |
| M2+ | 18 | RB9 | 3 | 10 | 3A |
| M2- | 24 | RB13 | 4 | 15 | 4A |

Input/Output

| Description | A/D | Pin# on PIC32 | Port on PIC32 | I/O |
| --- | --- | --- | --- | --- |
| Phototransistor (IR) | Digital | 6 | RB2 | Input |
| Line Detector | Digital | 7 | RB3 | Input |
| Debug Output | Digital | 26 | RB15 | Output |

SPI Pins Configuration

| Description | Pin# on PIC32 | Port on PIC32 |
| --- | --- | --- |
| SS | 2 | RA0 |
| SDO | 3 | RA1 |
| SDI | 17 | RB8 |
| SCK | 25 | RB14 |

Design Calculations

Choosing H-bridge - SN754410:

A coil of the DC motor typically has resistance 8.5Ω (by Digital Multimeter measurement), and draws approximately 588mA at 5V, steady state. If it draws as much as 588mA, the voltage will drop below 5V, to at least 3V, which corresponds to 353mA using 8.5Ω motor. Actually the voltage will be higher than 3V, and however the voltage drops the current can be safely supplied. Therefore, the SN754410-driven H-bridge can drive the coils.

Resistor Value for Amplifying Circuit

The reference voltage for the first-stage amplifier (using Op-Amp) is 2.5V, so the output of the first-stage amplifier is swinging from 2.5V to 0V. Then the second-stage thresholding (using comparator) uses the reference voltage of around 1.25V. Resistor values are found during field testing.

Mechanical Hardware Design

For Lab 8, we used onshape to design and create a virtual assembly of all our components. The main base for our mobile platform was laser cut out of ¼” thick birch plywood, with the remaining components fabricated using 3D printing. To make assembly easier, all of the hardware connections were standardized to use M4 screws. Starting with the motor clamps, these were 3D printed to roughly fit the shape of the provided motors and to clamp them down using the screws. Next, we also had to design a custom roller ball bearing caster for the front, so that the entire base can be supported. After finding a spare ball bearing, we designed the housing to not only accommodate it, but to also hold the TRFT5000 line sensor that we were using for line detection.  Based on the datasheet, the maximal range of the sensor is around 15 mm, which is less than the height of our base; hence why we added the sensor to the front caster assembly. Finally, for the base laser cut drawing, we assembled the components together and edited the base in context to make the proper cutouts/holes for each component. Generally speaking, due to the temporary nature of this lab, miscellaneous components (breadboards, wires, etc.) were fastened to the top using tape.

Software Design

DCMotorService controls 2 DC motors, and takes ES_MOTOR_ACTION_CHANGE event as a trigger for changing PWM states for the DC motors. It takes the desired speed and direction for both motors from the variables stored using the function MotorCommandWrapper called by the caller service (mainly MainLogicFSM) and configure a drive-brake control by setting PWM with the method used in Lab 7.

Beacon detection is done by simply checking if there is a rising edge coming from the phototransistor module (IR input) using EventCheckers. If so, it will then post an ES_BEACON_DETECTED event to MainLogicFSM. This method works because no other IR poise is present on the field, and once there is strong enough IR signal, the program believes a beacon has been detected.

CommandRetrieveService is responsible for periodically polling the SPI-based CommandGenerator and notifying MainLogicFSM when a new command becomes available. This service uses a periodic timer (COMMAND_SPI_TIMER) to initiate polling at fixed intervals. On each timeout: If 0xFF is received, a flag (SawNewCommandFlag) is set. On the subsequent query, if a valid command byte is received: The byte is verified against a lookup table in CommonDefinitions. If valid and different from the previously processed command, an ES_COMMAND_RETRIEVED(commandByte) event is posted to MainLogicFSM. Repeated old command values are ignored to prevent duplicate event posting.

CommonDefinitions.c/CommonDefinitions.h centralizes all shared system constants, command encodings, hardware configuration parameters to ensure consistency and reduce duplication across all motor control services.

Ports.c centralizes all microcontroller pin configuration and low-level digital I/O access, ensuring that hardware-specific TRIS/ANSEL setup and signal reads are abstracted cleanly from higher-level services.

MainLogicFSM is a finite state machine that controls the logic of the program. Refer to the state diagram below:

State Diagram of MainLogicFSM:

Pseudocode

1). MainLogicFSM:

Description

Pseudocode for MainLogicService based on the state diagram.

Handles command-driven motion modes and timer-based completion.

Notes

State Summary

- Stopped: idle, waits for commands

- SimpleMoving: executes open-loop moves (rotations/drives)

- SearchingForTape: drive forward until tape detected or timeout

- AligningWithBeacon: spin to align to beacon or timeout

Event

ES_COMMAND_RETRIEVED(commandByte)

ES_TAPE_DETECTED

ES_BEACON_DETECTED

ES_TIMEOUT(SIMPLE_MOVE_TIMER)

ES_TIMEOUT(TAPE_SEARCH_TIMER)

ES_TIMEOUT(BEACON_ALIGN_TIMER)

Action

MotorCommandWrapper(speedLeft, speedRight, dirLeft, dirRight)

Initialize SIMPLE_MOVE_TIMER

Initialize TAPE_SEARCH_TIMER

Initialize BEACON_ALIGN_TIMER

State Chart

Init -> Stopped

Stopped --ES_COMMAND_RETRIEVED(simple moves)--> SimpleMoving

Stopped --ES_COMMAND_RETRIEVED(tape search)--> SearchingForTape

Stopped --ES_COMMAND_RETRIEVED(beacon align)--> AligningWithBeacon

SimpleMoving --ES_TIMEOUT(SIMPLE_MOVE_TIMER)--> Stopped

SearchingForTape --ES_TAPE_DETECTED--> Stopped

SearchingForTape --ES_TIMEOUT(TAPE_SEARCH_TIMER)--> Stopped

AligningWithBeacon --ES_BEACON_DETECTED--> Stopped

AligningWithBeacon --ES_TIMEOUT(BEACON_ALIGN_TIMER)--> Stopped

MainLogicService

InitMainLogicService

Initialize ports via Ports.c/Ports.h

InitBeaconInputPin()

InitTapeSensorPin()

InitCommandSPIPins()

Set CurrentState = Stopped

MotorCommandWrapper(0,0,FORWARD,FORWARD)

RunMainLogicService

SWITCH(CurrentState)

State: Stopped

On ES_COMMAND_RETRIEVED(commandByte)

CASE commandByte

0x00: MotorCommandWrapper(0, 0, FORWARD, FORWARD)

0x02: RotateCW90(); CurrentState = SimpleMoving

0x03: RotateCW45(); CurrentState = SimpleMoving

0x04: RotateCCW90(); CurrentState = SimpleMoving

0x05: RotateCCW45(); CurrentState = SimpleMoving

0x08: DriveForwardHalf(); CurrentState = SimpleMoving

0x09: DriveForwardFull(); CurrentState = SimpleMoving

0x10: DriveReverseHalf(); CurrentState = SimpleMoving

0x11: DriveReverseFull(); CurrentState = SimpleMoving

0x20: AlignWithBeacon(); CurrentState = AligningWithBeacon

0x40: SearchForTape(); CurrentState = SearchingForTape

END CASE

State: SimpleMoving

On ES_TIMEOUT(SIMPLE_MOVE_TIMER)

MotorCommandWrapper(0,0,FORWARD,FORWARD)

CurrentState = Stopped

On ES_COMMAND_RETRIEVED(0x00)

MotorCommandWrapper(0,0,FORWARD,FORWARD)

PostMainLogicFSM(ThisEvent);

CurrentState = Stopped;

State: SearchingForTape

On ES_TAPE_DETECTED

MotorCommandWrapper(0,0,FORWARD,FORWARD)

CurrentState = Stopped

On ES_TIMEOUT(TAPE_SEARCH_TIMER)

MotorCommandWrapper(0,0,FORWARD,FORWARD)

Print("Tape Search Failed")

CurrentState = Stopped

On ES_COMMAND_RETRIEVED(0x00)

MotorCommandWrapper(0,0,FORWARD,FORWARD)

PostMainLogicFSM(ThisEvent);

CurrentState = Stopped

State: AligningWithBeacon

On ES_BEACON_DETECTED

MotorCommandWrapper(0,0,FORWARD,FORWARD)

CurrentState = Stopped

On ES_TIMEOUT(BEACON_ALIGN_TIMER)

MotorCommandWrapper(0,0,FORWARD,FORWARD)

Print("Alignment Failed")

CurrentState = Stopped

On ES_COMMAND_RETRIEVED(0x00)

MotorCommandWrapper(0,0,FORWARD,FORWARD)

PostMainLogicFSM(ThisEvent);

CurrentState = Stopped

SimpleMoves helpers (open-loop)

RotateCW90()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE)

Initialize SIMPLE_MOVE_TIMER to 6000 ms

RotateCW45()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE)

Initialize SIMPLE_MOVE_TIMER to 3000 ms

RotateCCW90()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, FORWARD)

Initialize SIMPLE_MOVE_TIMER to 6000 ms

RotateCCW45()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, FORWARD)

Initialize SIMPLE_MOVE_TIMER to 3000 ms

DriveForwardHalf()

MotorCommandWrapper(HALF_SPEED, HALF_SPEED, FORWARD, FORWARD)

Initialize SIMPLE_MOVE_TIMER to 1000 ms (optional: remove for continuous)

DriveForwardFull()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, FORWARD)

Initialize SIMPLE_MOVE_TIMER to 1000 ms (optional: remove for continuous)

DriveReverseHalf()

MotorCommandWrapper(HALF_SPEED, HALF_SPEED, REVERSE, REVERSE)

Initialize SIMPLE_MOVE_TIMER to 1000 ms (optional: remove for continuous)

DriveReverseFull()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, REVERSE)

Initialize SIMPLE_MOVE_TIMER to 1000 ms (optional: remove for continuous)

SearchForTape()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, FORWARD)

Initialize TAPE_SEARCH_TIMER

AlignWithBeacon()

MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE)

Initialize BEACON_ALIGN_TIMER

2). Event Checker for beacon and tape:

Description

Pseudocode for event checkers related to beacon, tape.

Notes

EventCheckers should detect transitions and post ES_* events to services.

Use static "last" variables to avoid repeated events while input stays low.

Event

ES_BEACON_DETECTED

ES_TAPE_DETECTED

Beacon EventChecker

Check for IR input I/O line (digital input) (initialized in Ports.c/Ports.h)

IF input transitions from LOW to HIGH

Post ES_BEACON_DETECTED to main logic

Tape EventChecker

Check for tape sensor input (digital or analog threshold) (initialized in Ports.c/Ports.h)

IF input transitions from LOW to HIGH

Post ES_TAPE_DETECTED to main logic

3). Receiveing command from command generator:

Description

Pseudocode for SPI command retrieval and posting command events.

Event

ES_COMMAND_RETRIEVED(commandByte)

InitCommandRetrieveService

Initialize flag to track when new command is ready

Configure SPI hardware as leader with appropriate timing settings

Set up chip select, data input, and data output pins

Enable SPI communication

Start timer to poll for commands every 2 seconds

Post initialization event

RunCommandRetrieveService

When timer expires:

Query the command generator for a new byte

If received 0xFF (new command ready flag):

Remember that next byte will be the new command

Otherwise if we previously saw the ready flag:

If the byte is a valid command:

If it differs from the last command posted:

Post the new command to the main logic state machine

Remember this command

Otherwise:

Log that an invalid command was received

Clear the new command ready flag

Restart the polling timer

QueryCommandGenerator

Send a dummy byte to clock out a response from the follower

Read and return the response byte

IsValidCommandByte

Check if the byte matches any entry in the valid commands lookup table

Return true if valid, false otherwise

4). Motor driving service:

Description

Pseudocode for controlling two DC motors (left/right) in one service.

Uses array-based motor state for clean, symmetric updates.

Notes

Guiding recommendation:

- Use ONE DCMotorService that owns both motors.

- Represent motors as an array of structs (index 0 = left, index 1 = right)

so the same code path configures/updates either motor.

- Provide a single wrapper API (MotorCommandWrapper) that sets both motors

and posts ES_MOTOR_ACTION_CHANGE to DCMotorService.

Data structures (module-level in DCMotorService):

MotorState {

speedTicks

direction

pwmOcModule

forwardPin

reversePin

}

MotorState Motors[2]  // Motors[LEFT], Motors[RIGHT]

Event

ES_MOTOR_ACTION_CHANGE(desiredSpeed)

InitDCMotorService

Set MyPriority

Initialize DesiredSpeed[LEFT_MOTOR] = 0, DesiredSpeed[RIGHT_MOTOR] = 0

Initialize DesiredDirection[LEFT_MOTOR] = FORWARD, DesiredDirection[RIGHT_MOTOR] = FORWARD

Call ConfigureDCMotorPins():

Configure RB4, RB5 (left motor), RB11, RB13 (right motor) as digital outputs

Initialize all motor pins to LOW

Map OC1 output to RB4 (left motor PWM)

Map OC2 output to RB11 (right motor PWM)

Call ConfigurePWM():

Configure Timer2 as PWM time base with PRESCALE_2

Set PR2 = PWM_PERIOD_TICKS

Configure OC1 and OC2 for PWM mode (OCM = 0b110)

Set initial duty cycle OC1RS = OC2RS = 0

Enable OC1 and OC2 modules

Start Timer2

Post ES_INIT event

MotorCommandWrapper(speedLeft, speedRight, dirLeft, dirRight)

DesiredSpeed[LEFT_MOTOR] = speedLeft

DesiredSpeed[RIGHT_MOTOR] = speedRight

DesiredDirection[LEFT_MOTOR] = dirLeft

DesiredDirection[RIGHT_MOTOR] = dirRight

Post ES_MOTOR_ACTION_CHANGE event to DCMotorService

RunDCMotorService

On ES_INIT:

No action needed (initialization done in Init function)

On ES_MOTOR_ACTION_CHANGE:

dutyCycle = MapSpeedToDutyCycle(DesiredSpeed[LEFT_MOTOR])

IF DesiredDirection[LEFT_MOTOR] == FORWARD

MOTOR_REVERSE_PIN_L = 0

OC1RS = dutyCycle

ELSE (REVERSE)

MOTOR_REVERSE_PIN_L = 1

OC1RS = PWM_PERIOD_TICKS - dutyCycle + 1

IF DesiredDirection[RIGHT_MOTOR] == FORWARD

MOTOR_REVERSE_PIN_R = 0

OC2RS = dutyCycle

ELSE (REVERSE)

MOTOR_REVERSE_PIN_R = 1

OC2RS = PWM_PERIOD_TICKS - dutyCycle + 1

MapSpeedToDutyCycle(desiredSpeed)

dutyCycle = desiredSpeed (direct mapping)

Clamp dutyCycle between DUTY_MIN_TICKS and DUTY_MAX_TICKS

RETURN dutyCycle

Source Code

1). MainLogicFSM:

/**************************************************************************** Module MainLogicFSM.c Revision 0.1 Description Main logic state machine for command-driven robot behavior. Notes States: - Stopped - SimpleMoving - SearchingForTape - AligningWithBeacon History When Who What/Why -------------- --- -------- 02/03/26 Tianyu Initial creation for Lab 8 main logic ****************************************************************************/ /*----------------------------- Include Files -----------------------------*/ #include "ES_Configure.h" #include "ES_Framework.h" #include "ES_Timers.h" #include "MainLogicFSM.h" #include "DCMotorService.h" #include "CommonDefinitions.h" #include "dbprintf.h" #include "Ports.h" #include "dbprintf.h" /*----------------------------- Module Defines ----------------------------*/ /*---------------------------- Module Functions ---------------------------*/ static void RotateCW90(void); static void RotateCW45(void); static void RotateCCW90(void); static void RotateCCW45(void); static void DriveForwardHalf(void); static void DriveForwardFull(void); static void DriveReverseHalf(void); static void DriveReverseFull(void); static void SearchForTape(void); static void AlignWithBeacon(void); /*---------------------------- Module Variables ---------------------------*/ static MainLogicState_t CurrentState; static uint8_t MyPriority; /*------------------------------ Module Code ------------------------------*/ /**************************************************************************** Function InitMainLogicFSM Parameters uint8_t : the priority of this service Returns bool, false if error in initialization, true otherwise Description Initializes the main logic state machine. Author Tianyu, 02/03/26 ****************************************************************************/ bool InitMainLogicFSM(uint8_t Priority) { ES_Event_t ThisEvent; MyPriority = Priority; /******************************************** Initialization code for ports and sensors *******************************************/ // TODO: Initialize ports via Ports.c/Ports.h InitBeaconInputPin(); InitTapeSensorPin(); InitCommandSPIPins(); InitDebugOutputPin(); CurrentState = Stopped; // Stop motors on startup MotorCommandWrapper(0, 0, FORWARD, FORWARD); ThisEvent.EventType = ES_INIT; if (ES_PostToService(MyPriority, ThisEvent) == true) { return true; } return false; } /**************************************************************************** Function PostMainLogicFSM Parameters ES_Event_t ThisEvent , the event to post to the queue Returns bool, false if the enqueue operation failed, true otherwise Description Posts an event to this state machine's queue Author Tianyu, 02/03/26 ****************************************************************************/ bool PostMainLogicFSM(ES_Event_t ThisEvent) { return ES_PostToService(MyPriority, ThisEvent); } /**************************************************************************** Function RunMainLogicFSM Parameters ES_Event_t : the event to process Returns ES_Event_t, ES_NO_EVENT if no error ES_ERROR otherwise Description State machine for command-driven robot behavior. Author Tianyu, 02/03/26 ****************************************************************************/ ES_Event_t RunMainLogicFSM(ES_Event_t ThisEvent) { ES_Event_t ReturnEvent; ReturnEvent.EventType = ES_NO_EVENT; // DB_printf("Current State is %d \r\n", CurrentState); switch (CurrentState) { case Stopped: if (ThisEvent.EventType == ES_COMMAND_RETRIEVED) { switch (ThisEvent.EventParam) { case CMD_STOP: MotorCommandWrapper(0, 0, FORWARD, FORWARD); break; case CMD_ROTATE_CW_90: DB_printf("State: Rotating CW 90 deg\r\n"); RotateCW90(); CurrentState = SimpleMoving; break; case CMD_ROTATE_CW_45: DB_printf("State: Rotating CW 45 deg\r\n"); RotateCW45(); CurrentState = SimpleMoving; break; case CMD_ROTATE_CCW_90: DB_printf("State: Rotating CCW 90 deg\r\n"); RotateCCW90(); CurrentState = SimpleMoving; break; case CMD_ROTATE_CCW_45: DB_printf("State: Rotating CCW 45 deg\r\n"); RotateCCW45(); CurrentState = SimpleMoving; break; case CMD_DRIVE_FWD_HALF: DB_printf("State: drive forwards half speed\r\n"); DriveForwardHalf(); CurrentState = SimpleMoving; break; case CMD_DRIVE_FWD_FULL: DB_printf("State: drive forwards full speed\r\n"); DriveForwardFull(); CurrentState = SimpleMoving; break; case CMD_DRIVE_REV_HALF: DB_printf("State: drive reverse half speed\r\n"); DriveReverseHalf(); CurrentState = SimpleMoving; break; case CMD_DRIVE_REV_FULL: DB_printf("State: drive reverse full speed\r\n"); DriveReverseFull(); CurrentState = SimpleMoving; break; case CMD_ALIGN_BEACON: DB_printf("State: aligning with beacon\r\n"); // If already HIGH, the ES_BEACON_DETECTED event will be posted immediately if( ReadBeaconInputPin() == true ) { ES_Event_t BeaconEvent; BeaconEvent.EventType = ES_BEACON_DETECTED; BeaconEvent.EventParam = 0; PostMainLogicFSM(BeaconEvent); } else{ // If not detected, act to look for beacon signal AlignWithBeacon(); } CurrentState = AligningWithBeacon; break; case CMD_SEARCH_TAPE: DB_printf("State: searching for tape \r\n"); // If already HIGH, the ES_TAPE_DETECTED event will be posted immediately if( ReadTapeSensorPin() == true ) { ES_Event_t TapeEvent; TapeEvent.EventType = ES_TAPE_DETECTED; TapeEvent.EventParam = 0; PostMainLogicFSM(TapeEvent); } else{ // If not detected, act to look for line detect signal SearchForTape(); } CurrentState = SearchingForTape; break; default: break; } } break; case SimpleMoving: if (ThisEvent.EventType == ES_TIMEOUT && ThisEvent.EventParam == SIMPLE_MOVE_TIMER) // movement timer expired after a set amount of time { DB_printf("Motor Timeout Received while moving\r\n"); MotorCommandWrapper(0, 0, FORWARD, FORWARD); CurrentState = Stopped; } else if (ThisEvent.EventType == ES_COMMAND_RETRIEVED) // while simple moving, new command received { DB_printf("New command received while moving\r\n"); CurrentState = Stopped; PostMainLogicFSM(ThisEvent); //go back to stopped list to take action on new command } break; case SearchingForTape: if (ThisEvent.EventType == ES_TAPE_DETECTED) // detected tape { DB_printf("Tape detected\r\n"); MotorCommandWrapper(0, 0, FORWARD, FORWARD); CurrentState = Stopped; } else if (ThisEvent.EventType == ES_TIMEOUT && ThisEvent.EventParam == TAPE_SEARCH_TIMER) // stop looking for tape after set time { MotorCommandWrapper(0, 0, FORWARD, FORWARD); DB_printf("Tape Search Failed: Timeout"); CurrentState = Stopped; } else if (ThisEvent.EventType == ES_COMMAND_RETRIEVED) // new command received while searching for tape { DB_printf("New command received while searching for tape\r\n"); CurrentState = Stopped; PostMainLogicFSM(ThisEvent); } break; case AligningWithBeacon: if (ThisEvent.EventType == ES_BEACON_DETECTED) // found direction of beacon { DB_printf("Found beacon\r\n"); MotorCommandWrapper(0, 0, FORWARD, FORWARD); // change speed CurrentState = Stopped; } else if (ThisEvent.EventType == ES_TIMEOUT && ThisEvent.EventParam == BEACON_ALIGN_TIMER) // set time passed, stop aligning towards beacon { MotorCommandWrapper(0, 0, FORWARD, FORWARD); DB_printf("Beacon Search Failed: Timeout"); CurrentState = Stopped; } else if (ThisEvent.EventType == ES_COMMAND_RETRIEVED) // new command received while aligning for beacon { DB_printf("New command received while aligning with beacon\r\n"); CurrentState = Stopped; PostMainLogicFSM(ThisEvent); } break; default: break; } return ReturnEvent; } /**************************************************************************** Function QueryMainLogicFSM Parameters None Returns MainLogicState_t: the current state of the main logic FSM Description Returns the current state of the main logic FSM Author Tianyu, 02/03/26 ****************************************************************************/ MainLogicState_t QueryMainLogicFSM(void) { return CurrentState; } /*----------------------------- Helper Functions --------------------------*/ /**************************************************************************** Function RotateCW90 Parameters None Returns None Description Open-loop 90 degree clockwise rotation. Author Tianyu, 02/03/26 ****************************************************************************/ static void RotateCW90(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE) // Initialize SIMPLE_MOVE_TIMER to 6000 ms MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE); ES_Timer_InitTimer(SIMPLE_MOVE_TIMER, SIMPLE_MOVE_90_MS); } /**************************************************************************** Function RotateCW45 Parameters None Returns None Description Open-loop 45 degree clockwise rotation. Author Tianyu, 02/03/26 ****************************************************************************/ static void RotateCW45(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE) // Initialize SIMPLE_MOVE_TIMER to 3000 ms MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE); ES_Timer_InitTimer(SIMPLE_MOVE_TIMER, SIMPLE_MOVE_45_MS); } /**************************************************************************** Function RotateCCW90 Parameters None Returns None Description Open-loop 90 degree counter-clockwise rotation. Author Tianyu, 02/03/26 ****************************************************************************/ static void RotateCCW90(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, FORWARD) // Initialize SIMPLE_MOVE_TIMER to 6000 ms MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, FORWARD); ES_Timer_InitTimer(SIMPLE_MOVE_TIMER, SIMPLE_MOVE_90_MS); } /**************************************************************************** Function RotateCCW45 Parameters None Returns None Description Open-loop 45 degree counter-clockwise rotation. Author Tianyu, 02/03/26 ****************************************************************************/ static void RotateCCW45(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, FORWARD) // Initialize SIMPLE_MOVE_TIMER to 3000 ms MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, FORWARD); ES_Timer_InitTimer(SIMPLE_MOVE_TIMER, SIMPLE_MOVE_45_MS); } /**************************************************************************** Function DriveForwardHalf Parameters None Returns None Description Drive forward at half speed (open-loop). Author Tianyu, 02/03/26 ****************************************************************************/ static void DriveForwardHalf(void) { // Pseudocode: // MotorCommandWrapper(HALF_SPEED, HALF_SPEED, FORWARD, FORWARD) // Optionally set SIMPLE_MOVE_TIMER MotorCommandWrapper(HALF_SPEED, HALF_SPEED, FORWARD, FORWARD); } /**************************************************************************** Function DriveForwardFull Parameters None Returns None Description Drive forward at full speed (open-loop). Author Tianyu, 02/03/26 ****************************************************************************/ static void DriveForwardFull(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, FORWARD) // Optionally set SIMPLE_MOVE_TIMER MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, FORWARD); } /**************************************************************************** Function DriveReverseHalf Parameters None Returns None Description Drive reverse at half speed (open-loop). Author Tianyu, 02/03/26 ****************************************************************************/ static void DriveReverseHalf(void) { // Pseudocode: // MotorCommandWrapper(HALF_SPEED, HALF_SPEED, REVERSE, REVERSE) // Optionally set SIMPLE_MOVE_TIMER MotorCommandWrapper(HALF_SPEED, HALF_SPEED, REVERSE, REVERSE); } /**************************************************************************** Function DriveReverseFull Parameters None Returns None Description Drive reverse at full speed (open-loop). Author Tianyu, 02/03/26 ****************************************************************************/ static void DriveReverseFull(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, REVERSE) // Optionally set SIMPLE_MOVE_TIMER MotorCommandWrapper(FULL_SPEED, FULL_SPEED, REVERSE, REVERSE); } /**************************************************************************** Function SearchForTape Parameters None Returns None Description Drive forward until tape detected or timeout. Author Tianyu, 02/03/26 ****************************************************************************/ static void SearchForTape(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, FORWARD) // Initialize TAPE_SEARCH_TIMER MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, FORWARD); // ES_Timer_InitTimer(TAPE_SEARCH_TIMER, TAPE_SEARCH_MS); } /**************************************************************************** Function AlignWithBeacon Parameters None Returns None Description Spin until beacon detected or timeout. Author Tianyu, 02/03/26 ****************************************************************************/ static void AlignWithBeacon(void) { // Pseudocode: // MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE) // Initialize BEACON_ALIGN_TIMER MotorCommandWrapper(FULL_SPEED, FULL_SPEED, FORWARD, REVERSE); ES_Timer_InitTimer(BEACON_ALIGN_TIMER, BEACON_ALIGN_MS); }

2).Event Checker for beacon and tape detect:

/**************************************************************************** Module EventCheckers.c Revision 0.1 Description Event checkers for beacon, tape, and keyboard input. Notes Use static variables to detect transitions. History When Who What/Why -------------- --- -------- 02/03/26 Tianyu Initial creation for Lab 8 event checkers ****************************************************************************/ /*----------------------------- Include Files -----------------------------*/ #include "ES_Configure.h" #include "ES_Framework.h" #include "ES_Events.h" #include "ES_PostList.h" #include "ES_ServiceHeaders.h" #include "ES_Port.h" #include "EventCheckers.h" #include "CommonDefinitions.h" #include "dbprintf.h" #include "Ports.h" /*------------------------------ Module Code ------------------------------*/ /**************************************************************************** Function Check4Keystroke Parameters None Returns bool: true if a new key was detected & posted Description checks to see if a new key from the keyboard is detected and, if so, retrieves the key and posts an ES_NEW_KEY event to TestHarnessService0 Notes The functions that actually check the serial hardware for characters and retrieve them are assumed to be in ES_Port.c Author J. Edward Carryer, 08/06/13, 13:48 ****************************************************************************/ bool Check4Keystroke(void) { if (IsNewKeyReady()) // new key waiting? { ES_Event_t ThisEvent; ThisEvent.EventType = ES_NEW_KEY; ThisEvent.EventParam = GetNewKey(); ES_PostAll(ThisEvent); return true; } return false; } /**************************************************************************** Function Check4TapeDetected Parameters None Returns bool: true if a new tape event was detected & posted Description Checks for a low-going transition on the tape sensor input. Author Tianyu, 02/03/26 ****************************************************************************/ bool Check4TapeDetected(void) { static bool LastTapeState = true; bool CurrentTapeState = ReadTapeSensorPin(); if ((CurrentTapeState == false) && (LastTapeState == true)) { ES_Event_t ThisEvent; ThisEvent.EventType = ES_TAPE_DETECTED; ThisEvent.EventParam = 0; PostMainLogicFSM(ThisEvent); LastTapeState = CurrentTapeState; return true; } LastTapeState = CurrentTapeState; return false; } /**************************************************************************** Function Check4CommandAvailable Parameters None Returns bool: true if a new command event was detected & posted Description Placeholder event checker. Command retrieval is handled by the CommandRetrieveService (SPI polling/interrupts). Author Tianyu, 02/03/26 ****************************************************************************/ bool Check4CommandAvailable(void) { // TODO: If moving command retrieval into an event checker, implement here. return false; } /**************************************************************************** Function Check4BeaconDetected Parameters None Returns bool: true if IR input is HIGH Description Beaon event checker. The IR beacon sensor is active HIGH. Author Tianyu, 02/04/26 ****************************************************************************/ bool Check4BeaconDetected(void) { static bool LastBeaconState = false; bool CurrentBeaconState = ReadBeaconInputPin(); if ((CurrentBeaconState == true) && (LastBeaconState == false)) { DEBUG_OUTPUT_PIN_LAT = 1; ES_Event_t ThisEvent; ThisEvent.EventType = ES_BEACON_DETECTED; ThisEvent.EventParam = 0; PostMainLogicFSM(ThisEvent); LastBeaconState = CurrentBeaconState; // printf("Posting ES_BEACON_DETECTED event.\r\n"); DEBUG_OUTPUT_PIN_LAT = 0; return true; } LastBeaconState = CurrentBeaconState; return false; }

3). Receiveing command from command generator:

/**************************************************************************** Module CommandRetrieveService.c Revision 0.1 Description Service for polling SPI and posting ES_COMMAND_RETRIEVED events when a new command is available from the CommandGenerator. Notes CommandGenerator is an SPI follower and SPI32 is the leader. Query behavior: - When a new command is ready, the next query returns 0xFF. - The query following that 0xFF returns the new command byte. - Subsequent queries return the same command byte until a new command arrives. This service should post ES_COMMAND_RETRIEVED(commandByte) to MainLogicService when a valid command byte is received after a 0xFF flag. History When Who What/Why -------------- --- -------- 02/03/26 Tianyu Initial creation for Lab 8 command retrieval ****************************************************************************/ /*----------------------------- Include Files -----------------------------*/ #include "ES_Configure.h" #include "ES_Framework.h" #include "ES_Timers.h" #include "CommandRetrieveService.h" #include "CommonDefinitions.h" #include "PIC32_SPI_HAL.h" #include "MainLogicFSM.h" #include "dbprintf.h" #include <xc.h> #include <sys/attribs.h> /*----------------------------- Module Defines ----------------------------*/ #define SPI_POLL_INTERVAL_MS 2000 SPI_Module_t Module = SPI_SPI1; /*---------------------------- Module Functions ---------------------------*/ static uint8_t ReadSPICommandByte(void); static bool IsValidCommandByte(uint8_t commandByte); /*---------------------------- Module Variables ---------------------------*/ static uint8_t MyPriority; static bool SawNewCommandFlag; static uint8_t LastCommand; uint8_t QueryCommandGenerator(void); /*------------------------------ Module Code ------------------------------*/ /* void __ISR(_SPI_1_VECTOR, IPL4SOFT) SPI1Handler(void){ uint8_t data = 0x0; data = (uint8_t) SPIOperate_ReadData(Module); IEC1CLR = _IEC1_SPI1RXIE_MASK ¦ _IEC1_SPI1TXIE_MASK ¦ _IEC1_SPI1EIE_MASK; CurrentCommand = data; } * */ /**************************************************************************** Function InitCommandRetrieveService Parameters uint8_t : the priority of this service Returns bool, false if error in initialization, true otherwise Description Initializes the SPI command retrieval service. Author Tianyu, 02/03/26 ****************************************************************************/ bool InitCommandRetrieveService(uint8_t Priority) { ES_Event_t ThisEvent; MyPriority = Priority; SawNewCommandFlag = false; /******************************************** Initialization code for SPI command retrieval *******************************************/ // TODO: Configure SPI32 as leader (mode, clock, chip select) // TODO: Initialize Ports/SPI pins via Ports.c if needed SPI_SamplePhase_t SamplePhase = SPI_SMP_MID; uint32_t DesiredClock_ns = 10000; SPI_Clock_t ClockIdle = SPI_CLK_HI; SPI_ActiveEdge_t ChosenEdge = SPI_FIRST_EDGE; SPI_XferWidth_t DataWidth = SPI_8BIT; SPI_PinMap_t SSPin = SPI_RPA0; // Chip Select Pin SPI_PinMap_t SDOPin = SPI_RPA1; // Chip Output Pin SPI_PinMap_t SDIPin = SPI_RPB8; // Chip Input Pin TRISAbits.TRISA0 = 0; TRISAbits.TRISA1 = 0; ANSELAbits.ANSA0 = 0; ANSELAbits.ANSA1 = 0; SPI1CONbits.SRXISEL = 0b01; SPISetup_BasicConfig(Module); SPISetup_SetLeader(Module, SamplePhase); SPISetup_SetBitTime(Module, DesiredClock_ns); SPISetup_MapSSOutput(Module, SSPin); SPISetup_MapSDOutput(Module, SDOPin); //SPISetup_MapSDInput(Module, SDIPin); SDI1R = 0b0100; TRISBbits.TRISB8 = 1; SPISetup_SetClockIdleState(Module, ClockIdle); SPISetup_SetActiveEdge(Module, ChosenEdge); SPISetup_SetXferWidth(Module, DataWidth); SPISetEnhancedBuffer(Module, true); SPISetup_EnableSPI(Module); /* Set up interrupt */ /* INTCONbits.MVEC = 1; IPC7bits.SPI1IP = 4; IPC7bits.SPI1IS = 0; IFS1CLR = _IFS1_SPI1RXIF_MASK; IEC1SET = _IEC1_SPI1RXIE_MASK; */ // Start a periodic poll timer if polling is used ES_Timer_InitTimer(COMMAND_SPI_TIMER, SPI_POLL_INTERVAL_MS); __builtin_enable_interrupts(); // Post the initial transition event ThisEvent.EventType = ES_INIT; if (ES_PostToService(MyPriority, ThisEvent) == true) { return true; } return false; } /**************************************************************************** Function PostCommandRetrieveService Parameters ES_Event_t ThisEvent , the event to post to the queue Returns bool, false if the enqueue operation failed, true otherwise Description Posts an event to this service's queue Author Tianyu, 02/03/26 ****************************************************************************/ bool PostCommandRetrieveService(ES_Event_t ThisEvent) { return ES_PostToService(MyPriority, ThisEvent); } /**************************************************************************** Function RunCommandRetrieveService Parameters ES_Event_t : the event to process Returns ES_Event_t, ES_NO_EVENT if no error ES_ERROR otherwise Description Service state machine for SPI command retrieval. Author Tianyu, 02/03/26 ****************************************************************************/ ES_Event_t RunCommandRetrieveService(ES_Event_t ThisEvent) { ES_Event_t ReturnEvent; ReturnEvent.EventType = ES_NO_EVENT; switch (ThisEvent.EventType) { case ES_INIT: // No action needed beyond initialization break; case ES_TIMEOUT: { if (ThisEvent.EventParam != COMMAND_SPI_TIMER) { break; } // Pseudocode: poll SPI follower for command bytes // Read byte from SPI (leader initiates query) // IF byte == 0xFF // SawNewCommandFlag = true // ELSE IF SawNewCommandFlag == true // IF byte is valid command // Post ES_COMMAND_RETRIEVED(commandByte) to MainLogicService // ELSE // Ignore or log invalid command // SawNewCommandFlag = false // ELSE // Ignore (repeated old command value) uint8_t commandByte = (uint8_t) QueryCommandGenerator(); DB_printf("Received Command byte: 0x%x\r\n", commandByte); if (commandByte == 0xFF) { SawNewCommandFlag = true; } else if (SawNewCommandFlag == true) { if (IsValidCommandByte(commandByte)) { ES_Event_t CommandEvent; if(commandByte != LastCommand) { CommandEvent.EventType = ES_COMMAND_RETRIEVED; CommandEvent.EventParam = commandByte; PostMainLogicFSM(CommandEvent); LastCommand = commandByte; } } else { DB_printf("Invalid command byte: 0x%x\r\n", commandByte); } SawNewCommandFlag = false; LastCommand = 0xFF; } ES_Timer_InitTimer(COMMAND_SPI_TIMER, SPI_POLL_INTERVAL_MS); break; } default: break; } return ReturnEvent; } /*----------------------------- Module Helpers ----------------------------*/ /**************************************************************************** Function ReadSPICommandByte Parameters None Returns uint8_t command byte read from SPI Description Reads a byte from the SPI follower. Author Tianyu, 02/03/26 ****************************************************************************/ uint8_t QueryCommandGenerator(void) { // TODO: Implement SPI read (leader initiates query) // Pseudocode: // Assert chip select // Transfer dummy byte and read response // Deassert chip select // Return response byte if(!SPI1STATbits.SPITBF) { SPIOperate_SPI1_Send8Wait(0xAA); } uint8_t data = (uint8_t) SPIOperate_ReadData(Module); return data; } /**************************************************************************** Function IsValidCommandByte Parameters uint8_t commandByte Returns bool, true if the command byte is valid Description Validates a command byte against the lookup table. Author Tianyu, 02/03/26 ****************************************************************************/ static bool IsValidCommandByte(uint8_t commandByte) { // TODO: Implement command validation using CommonDefinitions lookup table bool returnVal = false; uint8_t index = 0; for(index; index < sizeof(validCommandBytes)/sizeof(validCommandBytes[0]); index++) { if(commandByte == validCommandBytes[index]) { returnVal = true; } } return returnVal; }

4). Motor Running Service:

/**************************************************************************** Module DCMotorService.c Revision 1.0.1 Description This service controls two DC motors (left/right) using PWM outputs. Notes When initializing the DC Motor Service: Configure & initialize I/O pins connected with DC motor for PWM Map OC output to I/O pin, connected to one of the motor control pins Disable the PWM Output Compare module Disable the PWM timer Set the TMRy prescale value and enable the time base by setting TON (TxCON<15>) =1 Set the PWM period by writing to the selected timer period register (PRy). IF (read from direction I/O pin, and it is LOW) Set the PWM duty cycle ticks by writing to the OCxRS register Set the other motor control pin to LOW ELSE Set the PWM duty cycle ticks to (PWM period – duty cycle ticks + 1) by writing to the OCxRS register Set the other motor control pin to HIGH Write the OCxR register with the initial duty cycle ticks. Configure the Output Compare module for one of two PWM Operation modes by writing to the Output Compare mode bits, OCM<2:0> (OCxCON<2:0>). Turn ON the Output Compare module Turn the timer to the PWM system on. On ES_MOTOR_ACTION_CHANGE event: Get desiredSpeed from event parameter Map the desired speed to duty cycle ticks IF (read from direction I/O pin, and it is LOW) Set the other motor control pin to LOW ELSE Set the new PWM duty cycle ticks to (PWM period – duty cycle ticks + 1) Set the other motor control pin to HIGH Clamp duty cycle ticks to safe range Write new duty cycle ticks to OCxRS History When Who What/Why -------------- --- -------- 01/21/26 Tianyu Updated for Lab 6 motor speed control 01/15/26 Tianyu Fixed position wrapping logic for unsigned type 01/14/26 Tianyu Initial creation for Lab 5 ****************************************************************************/ /*----------------------------- Include Files -----------------------------*/ #include "ES_Configure.h" #include "ES_Framework.h" #include "ES_Timers.h" #include "DCMotorService.h" #include "CommonDefinitions.h" #include "dbprintf.h" #include <xc.h> /*----------------------------- Module Defines ----------------------------*/ // Port definitions #define MOTOR_FORWARD_PIN_L LATBbits.LATB4 #define MOTOR_REVERSE_PIN_L LATBbits.LATB5 //#define DIRECTION_PIN PORTBbits.RB8 // Direction input pin //Right Wheel Port definitions #define MOTOR_FORWARD_PIN_R LATBbits.LATB11 #define MOTOR_REVERSE_PIN_R LATBbits.LATB13 // PWM configuration (period defined in CommonDefinitions.h) #define INITIAL_DUTY_TICKS 0 // Initial duty cycle in ticks #define ENABLE_POT_AD /*---------------------------- Module Functions ---------------------------*/ /* Prototypes for private functions for this service */ static void ConfigureTimeBase(uint8_t prescale); static void ConfigurePWM(void); static void ConfigureDCMotorPins(void); static uint16_t MapSpeedToDutyCycle(uint16_t desiredSpeed); /*---------------------------- Module Variables ---------------------------*/ // Module level Priority variable static uint8_t MyPriority; static uint16_t DesiredSpeed[2]; static uint8_t DesiredDirection[2]; /*------------------------------ Module Code ------------------------------*/ /**************************************************************************** Function InitDCMotorService Parameters uint8_t : the priority of this service Returns bool, false if error in initialization, true otherwise Description Initializes the DC Motor Service Author Tianyu, 01/14/26 ****************************************************************************/ bool InitDCMotorService(uint8_t Priority) { ES_Event_t ThisEvent; MyPriority = Priority; DesiredSpeed[LEFT_MOTOR] = 0; DesiredSpeed[RIGHT_MOTOR] = 0; DesiredDirection[LEFT_MOTOR] = FORWARD; DesiredDirection[RIGHT_MOTOR] = FORWARD; /******************************************** Initialization code for DC Motor Control system *******************************************/ // Initialize Output Compare Pins used // TODO: Expand ConfigureDCMotorPins for both motors ConfigureDCMotorPins(); // Configure PWM module (includes timer configuration) ConfigurePWM(); // Post the initial transition event ThisEvent.EventType = ES_INIT; if (ES_PostToService(MyPriority, ThisEvent) == true) { return true; } else { return false; } } /**************************************************************************** Function PostDCMotorService Parameters ES_Event_t ThisEvent, the event to post to the queue Returns bool false if the Enqueue operation failed, true otherwise Description Posts an event to this state machine's queue Notes Author Tianyu, 01/14/26 ****************************************************************************/ bool PostDCMotorService(ES_Event_t ThisEvent) { return ES_PostToService(MyPriority, ThisEvent); } /**************************************************************************** Function RunDCMotorService Parameters ES_Event_t : the event to process Returns ES_Event_t, ES_NO_EVENT if no error ES_ERROR otherwise Description Handles events to control the DC motor Author Tianyu, 01/14/26 ****************************************************************************/ ES_Event_t RunDCMotorService(ES_Event_t ThisEvent) { ES_Event_t ReturnEvent; ReturnEvent.EventType = ES_NO_EVENT; // Assume no errors switch (ThisEvent.EventType) { case ES_INIT: // Initialization already done in Init function break; case ES_MOTOR_ACTION_CHANGE: { // Pseudocode: // FOR each motor in Motors[] // Map desired speed to duty ticks // Clamp duty ticks to safe range // IF motor.direction == FORWARD // Set forward pin HIGH, reverse pin LOW // Write duty ticks to OCxRS // ELSE (REVERSE) // Set forward pin LOW, reverse pin HIGH // Write duty ticks to OCxRS (or inverted if hardware requires) // END FOR uint16_t dutyCycle = MapSpeedToDutyCycle(DesiredSpeed[LEFT_MOTOR]); if (DesiredDirection[0] == 0) { MOTOR_REVERSE_PIN_L = 0; OC1RS = dutyCycle; DB_printf("dutyCycle left 0:%u\r\n", dutyCycle); } else { MOTOR_REVERSE_PIN_L = 1; OC1RS = PWM_PERIOD_TICKS - dutyCycle + 1; DB_printf("dutyCycle left 1:%u\r\n", OC1RS); } // Hardware motor already inversed for right motor, when forward means same current go through left and right motor if (DesiredDirection[1] == 0) { MOTOR_REVERSE_PIN_R = 0; OC2RS = dutyCycle; DB_printf("dutyCycle right 0:%u\r\n", dutyCycle); } else { MOTOR_REVERSE_PIN_R = 1; OC2RS = PWM_PERIOD_TICKS - dutyCycle + 1; DB_printf("dutyCycle right 1:%u\r\n", OC2RS); } break; } default: break; } return ReturnEvent; } /**************************************************************************** Function MotorCommandWrapper Parameters uint16_t speedLeft, speedRight uint8_t dirLeft, dirRight Returns None Description Writes desired speeds/directions into module variables and posts ES_MOTOR_ACTION_CHANGE events to DCMotorService. Author Tianyu, 02/03/26 ****************************************************************************/ void MotorCommandWrapper(uint16_t speedLeft, uint16_t speedRight, uint8_t dirLeft, uint8_t dirRight) { ES_Event_t ThisEvent; DesiredSpeed[LEFT_MOTOR] = speedLeft; DesiredSpeed[RIGHT_MOTOR] = speedRight; DesiredDirection[LEFT_MOTOR] = dirLeft; DesiredDirection[RIGHT_MOTOR] = dirRight; ThisEvent.EventType = ES_MOTOR_ACTION_CHANGE; ThisEvent.EventParam = 0; PostDCMotorService(ThisEvent); DB_printf("DesiredSpeed:%u %u, DesiredDirection: %u %u\r\n", DesiredSpeed[0], DesiredSpeed[1], DesiredDirection[0], DesiredDirection[1]); } /*************************************************************************** Private Functions ***************************************************************************/ /**************************************************************************** Function ConfigureTimeBase Parameters None Returns None Description Configures the timer used as the time base for PWM operation Author Tianyu, 01/21/26 ****************************************************************************/ static void ConfigureTimeBase(uint8_t prescale) { // Clear the ON control bit to disable the timer T2CONbits.ON = 0; // Clear teh TCS control bit to select the internal PBCLK source T2CONbits.TCS = 0; // Select the desired timer input clock prescale T2CONbits.TCKPS = PrescaleLookup[prescale]; // Clear the timer register TMRx TMR2 = 0; // Enable the timer by setting the ON control bit T2CONbits.ON = 1; } /**************************************************************************** Function ConfigurePWM Parameters None Returns None Description Configures the PWM Output Compare module for PWM operation Notes This function configures both the timer base and the Output Compare module. The timer configuration is done first to ensure proper initialization order. Author Tianyu, 01/21/26 ****************************************************************************/ static void ConfigurePWM(void) { // Step 1: Configure the timer base (must be done before OC config) ConfigureTimeBase(PRESCALE_2); // Keep timer off during configuration to avoid unintended pulses T2CONbits.ON = 0; // Disable the PWM Output Compare module before configuration OC1CONbits.ON = 0; OC2CONbits.ON = 0; // Set the PWM period by writing to the timer period register PR2 = PWM_PERIOD_TICKS; // Write the OCxR register with the initial duty cycle OC1RS = INITIAL_DUTY_TICKS; // Configure the Output Compare module for PWM mode OC1CONbits.OCM = 0b110; // PWM mode on OCx; Fault pin disabled // Turn ON the Output Compare module OC1CONbits.ON = 1; OC2CONbits.OCM = 0b110; // PWM Mode OC2CONbits.OCTSEL = 0; // Also use Timer 2 OC2RS = INITIAL_DUTY_TICKS; OC2CONbits.ON = 1; // Start the timer after PWM configuration is complete TMR2 = 0; // Clear timer register for clean start T2CONbits.ON = 1; } /**************************************************************************** Function ConfigureDCMotorPins Parameters None Returns None Description Configures the I/O pins for DC motor control as outputs Author Tianyu, 01/21/26 ****************************************************************************/ static void ConfigureDCMotorPins(void) { // Configure pins as digital outputs (all pins here don't have analog functions) TRISBbits.TRISB4 = 0; // MOTOR_FORWARD as output TRISBbits.TRISB5 = 0; // MOTOR_REVERSE as output TRISBbits.TRISB11 = 0; // MOTOR_FORWARD_R as output TRISBbits.TRISB13 = 0; // MOTOR_REVERSE_R as output // Initialize all pins to low MOTOR_FORWARD_PIN_L = 0; MOTOR_REVERSE_PIN_L = 0; MOTOR_FORWARD_PIN_R = 0; MOTOR_REVERSE_PIN_R = 0; // Map OC1 output to RB4 RPB4R = 0b0101; // Map OC2 output to RB11 RPB11R = 0b0101; } /**************************************************************************** Function MapSpeedToDutyCycle Parameters uint16_t desiredSpeed: desired speed value (0 to ADC_MAX_VALUE) Returns uint16_t: duty cycle value (0 to PWM_PERIOD_TICKS), clamped to safe range Description Maps the desired speed from the ADC range to PWM duty cycle range and clamps the result to ensure it stays within valid bounds. Notes Uses ADC_MAX_VALUE from CommonDefinitions.h as the source of truth for the ADC range to ensure consistency across services. Author Tianyu, 01/21/26 ****************************************************************************/ static uint16_t MapSpeedToDutyCycle(uint16_t desiredSpeed) { // Map the desired speed to duty cycle // desiredSpeed range: [0, ADC_MAX_VALUE] → dutyCycle range: [0, PWM_PERIOD_TICKS] // uint32_t dutyCycle = ((uint32_t)desiredSpeed * PWM_PERIOD_TICKS) / ADC_MAX_VALUE; uint16_t dutyCycle = desiredSpeed; // Clamp duty cycle to safe range if (dutyCycle > DUTY_MAX_TICKS) { dutyCycle = DUTY_MAX_TICKS; } else if (dutyCycle < DUTY_MIN_TICKS) { dutyCycle = DUTY_MIN_TICKS; } return (uint16_t)dutyCycle; } /*------------------------------- Footnotes -------------------------------*/ /*------------------------------ End of file ------------------------------*/